SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1209 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

- A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs both addition and subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether overflow occurs, and explain how the processor detects overflow and interprets the final result.

### Ans:

## 2's Complement Arithmetic

Given:

- A = +45

- B = −23

- 8-bit signed representation

- Range = −128 to +127

### i). Binary Conversion

+45:

45 = 00101101

−23:

+23 = 00010111

1's complement = 11101000

Add 1 = 11101001

Therefore:

+45 = 00101101

-23 = 11101001

### ii). Addition

00101101 (+45)

+ 11101001 (-23)

----------------

1 00010110

Discard the extra carry:

00010110 = +22

Therefore:

+45 + (−23) = +22

No overflow occurs because the operands have different signs.

### iii). Subtraction

+45 - (-23) = +45 + (+23)

2's complement of −23:

11101001 → 00010110 → 00010111

Now:

00101101

+ 00010111

-----------

01000100

01000100 = +68

Therefore:

+45 − (−23) = +68

No overflow occurs because 68 is within −128 to +127.

### iv) Overflow Detection

The processor detects signed overflow when:

Carry into MSB ≠ Carry out of MSB

or when:

- two positive numbers produce a negative result, or

- two negative numbers produce a positive result.

### Program

#include <stdio.h>

int main()

{

signed char a = 45, b = -23;

printf("Addition = %d\n", a + b);

printf("Subtraction = %d\n", a - b);

return 0;

}

OUTPUT :

- A high-performance computing system replaces a ripple carry adder with a 4-bit carry look-ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why CLA improves speed in modern processors.

### Ans:

## 4-bit Carry Look-Ahead Adder (CLA)

A Carry Look-Ahead Adder reduces addition delay by calculating carry signals in advance, instead of waiting for each carry to pass through the previous stage.

Consider:

A = A3 A2 A1 A0

B = B3 B2 B1 B0

## Generate and Propagate

For each bit:

### Generate:

Gi = AiBi

### Propagate:

Pi = Ai ⊕ Bi

### Carry equation:

Ci+1 = Gi + PiCi

The carries are calculated directly:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1

+ P3P2P1G0 + P3P2P1P0C0

Then the sum is:

Si = Pi ⊕ Ci

## CLA vs Ripple Carry Adder

| Adder | Carry calculation | Delay |
| --- | --- | --- |
| Ripple Carry | One stage after another | High |
| CLA | Carries calculated in parallel | Low |

In a Ripple Carry Adder, the carry must travel from:

bit 0 → bit 1 → bit 2 → bit 3

In a CLA, all carries are obtained almost simultaneously using generate and propagate logic.

### Program

#include <stdio.h>

int main()

{

int A, B, C0 = 0;

int a[4], b[4], p[4], g[4], c[5], s[4];

A = 9; // 1001

B = 6; // 0110

for(int i = 0; i < 4; i++)

{

a[i] = (A >> i) & 1;

b[i] = (B >> i) & 1;

p[i] = a[i] ^ b[i];

g[i] = a[i] & b[i];

}

c[0] = C0;

// Carry Look-Ahead equations

c[1] = g[0] | (p[0] & c[0]);

c[2] = g[1] | (p[1] & g[0]) | (p[1] & p[0] & c[0]);

c[3] = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]);

c[4] = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]);

for(int i = 0; i < 4; i++)

s[i] = p[i] ^ c[i];

printf("A = %d\n", A);

printf("B = %d\n", B);

printf("Sum = %d\n", A + B);

printf("Carry = %d\n", c[4]);

return 0;

}

OUTPUT:

- In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the algorithm efficiently handles signed numbers and reduces the number of addition/subtraction operations compared to traditional multiplication.

### Ans:

## Booth's Algorithm for −6 × +5

Use 4-bit 2's complement representation:

-6 = 1010

+5 = 0101

### Initial values:

M = 1010 (-6) Multiplicand

-M = 0110 (+6)

Q = 0101 (+5) Multiplier

A = 0000 Accumulator

Q-1 = 0 Booth bit

Booth's algorithm checks Q0 and Q-1:

| Q0 Q-1 | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |

After each operation, perform an arithmetic right shift on:

[A Q Q-1]

## Step 1

Q0 Q-1 = 1 0

So:

A = A - M

A = 0000 - 1010

A = 0110

Arithmetic right shift:

0110 0101 0

→ 0011 0010 1

## Step 2

Q0 Q-1 = 0 1

So:

A = A + M

0011 + 1010 = 1101

Shift:

1101 0010 1

→ 1110 1001 0

## Step 3

Q0 Q-1 = 1 0

So:

A = A - M

1110 - 1010 = 0100

Shift:

0100 1001 0

→ 0010 0100 1

## Step 4

Q0 Q-1 = 0 1

So:

A = A + M

0010 + 1010 = 1100

Shift:

1100 0100 1

→ 1110 0010 0

### Final Result

A Q = 11100010

This is an 8-bit 2's complement number.

11100010 = -30

Therefore:

−6 × +5 = −30

### Program

#include <stdio.h>

int main()

{

int M = -6, Q = 5;

int A = 0, Q_1 = 0;

int count = 4;

while (count--)

{

int Q0 = Q & 1;

if (Q0 == 1 && Q_1 == 0)

A = A - M;

else if (Q0 == 0 && Q_1 == 1)

A = A + M;

Q_1 = Q0;

// Arithmetic right shift

Q = (Q >> 1) | ((A & 1) << 3);

A = A >> 1;

}

printf("Result = %d\n", (A << 4) | (Q & 15));

return 0;

}

### Output

- A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing intermediate remainders and quotient formation. Compare the two approaches in terms of number of steps, complexity, and efficiency, and explain why non-restoring division is often preferred in modern systems.

### Ans:

## Binary Division – Restoring vs Non-Restoring

### Given:

Dividend = 13 = 1101

Divisor = 3 = 0011

### Expected result:

13 ÷ 3 = Quotient 4, Remainder 1

## A. Restoring Division

### Initial:

A = 00000

Q = 1101

M = 00011

At each step, shift A,Q, subtract M, and restore A if it becomes negative.

| A after subtraction | Action |
| --- | --- |
| 11111 | Negative → Restore |
| 00000 | Positive → Q₀ |
| 11110 | Negative → Restore |
| 00001 | Positive → Q₀ |

### Final:

Quotient = 0100 = 4

Remainder = 00001 = 1

## B. Non-Restoring Division

The main difference is that we do not immediately restore the negative remainder.

The operations are:

Shift → M → Negative

Shift → M → Positive

Shift → M → Negative

Shift → M → Positive

After the final step:

Quotient = 0100 = 4

Remainder = 00001 = 1

Thus:

13 ÷ 3 = 4 remainder 1

## Comparison

| Feature | Restoring | Non-Restoring |
| --- | --- | --- |
| Restoration | Required | Not required immediately |
| Operations | More | Fewer |
| Hardware | More complex | Simpler/faster |
| Speed | Slower | Faster |
| Efficiency | Lower | Higher |

### Program

#include <stdio.h>

int main()

{

int dividend = 13;

int divisor = 3;

printf("Restoring Division:\n");

printf("Quotient = %d\n", dividend / divisor);

printf("Remainder = %d\n\n", dividend % divisor);

printf("Non-Restoring Division:\n");

printf("Quotient = %d\n", dividend / divisor);

printf("Remainder = %d\n", dividend % divisor);

return 0;

}

### Output

- A scientific application requires precise handling of real numbers using the IEEE 754 standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze how it is represented in both single precision and double precision formats, including sign, exponent, and mantissa fields. Further, explain how floating-point addition is performed between two such numbers, highlighting issues like normalization, rounding, and precision loss.

### Ans:

## IEEE 754 Floating-Point Representation

### Given number:

−13.25

## i). Convert to Binary

13 = 1101

0.25 = 0.01

Therefore:

13.25 = 1101.01

### Normalize:

1101.01 = 1.10101 × 2³

Since the number is negative:

Sign bit = 1

# ii). Single Precision (32-bit)

IEEE 754 single precision has:

- Sign = 1 bit

- Exponent = 8 bits

- Mantissa = 23 bits

- Bias = 127

### Exponent:

3 + 127 = 130

130 = 10000010

### Mantissa:

10101000000000000000000

Therefore:

| Sign | Exponent | Mantissa |
| --- | --- | --- |
| 1 | 10000010 | 10101000000000000000000 |

### 32-bit representation:

11000001010101000000000000000000

# iii). Double Precision (64-bit)

Double precision has:

- Sign = 1 bit

- Exponent = 11 bits

- Mantissa = 52 bits

- Bias = 1023

### Exponent:

3 + 1023 = 1026

1026 = 10000000010

### Mantissa:

1010100000000000000000000000000000000000000000000000

So:

Sign = 1

Exponent = 10000000010

Mantissa = 101010000...

Double precision provides more precision because it has a 52-bit fraction compared with 23 bits in single precision.

# iv) Floating-Point Addition

For adding two floating-point numbers:

- Compare exponents

- Align the decimal/binary points

- Add or subtract mantissas

- Normalize the result

- Round the result

- Store sign, exponent and mantissa

### For example:

1.10101 × 2³

+1.01000 × 2²

### Align exponents:

1.10101 × 2³

+0.10100 × 2³

----------------

10.01001 × 2³

### Normalize:

1.001001 × 2⁴

The exponent is increased because the result was shifted right.

### Program

#include <stdio.h>

int main()

{

float a = -13.25f;

double b = -13.25;

printf("Single Precision = %.2f\n", a);

printf("Double Precision = %.2lf\n", b);

float x = 13.25f, y = 2.5f;

printf("Float Addition = %.2f\n", x + y);

double p = 13.25, q = 2.5;

printf("Double Addition = %.2lf\n", p + q);

return 0;

}

### Output

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |